National Instruments PCI-GPIB Driver Installation Guide for Windows NT 4.0 and Windows 2000

- Exit all running programs (except Explorer) before start the installation.

- Make sure you have installed a National Instruments’ PCI-GPIB board in your system and connected to a CEQ (instrument or simulator).

- If you already have CEQ 2000 installed in your system (WinNT), remove the CEQPCINT driver from the Device Manager (contact Eric Chiang (x8658) for the detail).

- Run ni488216.exe.

- Press Setup to extract the installation files.

- Press Yes to accept the license.

- Do not register. Press Next to continue.

- Select the install directory. The default is C:\Program Files\National Instruments\NI-488.2. Press Next to continue.

- Select the install components. We recommend Driver and Applications only.

- Press Next to start installation.

- After finish copying files, press Finish to complete the installation.

- Press OK to reboot the system (required).

- After system starts up, the Wizard will guide you through the initialization. Click on “Verify your hardware and software installation”. You need to have a PCI-GPIB board installed and connected to a CEQ system.

- After Wizard verified your interface, press Exit to continue.

- Continue on to “Communicate with your instrument”.

- Click on X to exit out this dialog. Do not click on Next.

- Click on Exit to complete the Wizard verification procedure.

- Change the CEQ.INI for CEQ 2000 system to interface with the National driver.

[CEQRUN]

VXDNAME=CEQNIGPIB